The Rising Sun Court: Buy List

The artificial sun light and the artificial candle. First prototype of each. Written 19 October 2026 for Dr. O.

No prices are listed because I cannot check them from here, so the Price column is yours to fill in. Nothing below has been tried on real parts, and I have not seen any listing. If a listing does not match what is written, do not buy it. Send me a screenshot of the listing and I will tell you.

Choices already made by you: nothing is soldered, the speakers are the same ones as Astra-9, the sun uses the ESP32-S3 board, the candles are flameless and artificial, and both products borrow the same cloud brain and fairies.

# Part 1: The Rising Sun light

## Buy now: the light test

| Bought | Part | What the listing must say | How many | Your choice or link | Price |
| --- | --- | --- | --- | --- | --- |
| [ ] | Controller board | You already have the ESP32-S3 N16R8 from the Holopod build. Use one of those. | 0 | Already have |  |
| [ ] | LED strip | 5V, WS2812B (or WS2812), "individually addressable". 60 LEDs per meter. Black or white backing. NOT 12V, and NOT a 4 wire "RGB" strip with no data line. | 1 meter |  |  |
| [ ] | Power supply | 5V, rated 4 amps or more, with a screw terminal, or a barrel plug and a matching adapter. 60 LEDs can draw 3.6 amps at full white. NOT a phone charger. | 1 |  |  |
| [ ] | Resistor | 330 ohm. Any small assortment kit has it. | 1 |  |  |
| [ ] | Capacitor | 1000 microfarad, 6.3V or higher. Also in most kits. | 1 |  |  |
| [ ] | Jumper wires | Male to female and male to male, a small assortment. | 1 set |  |  |
| [ ] | Breadboard or screw terminal blocks | So nothing is soldered for the first try. | 1 |  |  |
| [ ] | USB cable | Must carry data, not only power. A charge-only cable is the usual reason a board does not show up. | 1 |  |  |

## Buy only if the first light flickers

| Bought | Part | What the listing must say | How many | Your choice or link | Price |
| --- | --- | --- | --- | --- | --- |
| [ ] | Level shifter | 74AHCT125, sold as "logic level shifter". The board sends 3.3V and some strips want 5V. | 1 |  |  |

## Buy for sound: no soldering

First choice is one all-in-one board, so there is nothing tiny to solder. The program that plays her voice and music is not written yet, so this is wired after the first light test.

| Bought | Part | What the listing must say | How many | Your choice or link | Price |
| --- | --- | --- | --- | --- | --- |
| [ ] | All-in-one audio board (first choice) | Search "ESP32-S3 audio development board speaker amplifier microphone". It must say: ESP32-S3 (not plain ESP32, not S2); a built-in speaker amplifier and a speaker connector; a built-in microphone or a microphone connector; header pins already soldered on with at least one spare free pin for the LED strip data wire; USB-C. Note the speaker plug type. | 1 |  |  |
| [ ] | Speakers | The same as Astra-9: use "Buy it again" on your Amazon order of September 15, 2026 (3 watt, 8 ohm mini speakers). The amplifier modules in that order are not needed. | 1 to 2 |  |  |
| [ ] | Second choice only: I2S amplifier | MAX98357A, header pins already attached in the photos, screw terminal speaker output. Skip any listing with loose pins in a bag. | 1 per speaker |  |  |
| [ ] | Second choice only: I2S microphone | INMP441, header pins already attached. | 1 |  |  |
| [ ] | Screw terminal blocks | The small green ones that held up on Astra-9, for the speaker wires. | 2 |  |  |

## Do not buy yet

The full length of strip, and the acrylic. How much depends on the acrylic shape, which is not decided.

The bigger power supply. It is sized from the final LED count: about 60 milliamps per LED at full white, plus some room.

A frosted diffuser sheet for the front of the acrylic. Pick it when the shape is decided.

# Part 2: The Rising Sun candle

One candle, one fairy, on a USB cord, with a button only. The fairy is your choice, so the fairy card is left blank. Everything here is my suggestion and none of it is decided.

| Bought | Part | What the listing must say | How many | Your choice or link | Price |
| --- | --- | --- | --- | --- | --- |
| [ ] | Controller and audio | The same all-in-one ESP32-S3 audio board as the light, or the one you already bought for it. The free pin drives the candle flame. | 1 |  |  |
| [ ] | Flame LED | One WS2812B single LED breakout board (about 10 mm), or a small NeoPixel "jewel" of 7 LEDs, warm white. Header pins or wires already attached. This lets the program make the flicker and the dim. Do NOT buy the self-flickering candle LEDs, because the program cannot control them. | 1 |  |  |
| [ ] | Speaker | The same Astra-9 3 watt, 8 ohm mini speaker. Use "Buy it again". | 1 |  |  |
| [ ] | Microphone | Only if the audio board has none: INMP441, pins already attached. | 0 or 1 |  |  |
| [ ] | Switch | A momentary push button with wires or a screw terminal, or a touch plate module (TTP223) with pins attached. The button is the on and off: on wakes the fairy, off puts the fairy to sleep. | 1 |  |  |
| [ ] | Power for the first one | A USB-C cable that carries data, and a 5V 2 amp USB wall charger. No battery yet. | 1 each |  |  |
| [ ] | The candle body | A flameless pillar candle with a hollow, translucent shell and an open bottom or a removable battery compartment, about 3 inch (75 mm) wide and 6 inch (150 mm) tall. Measure before buying: the board, speaker and wires need room inside. A frosted acrylic or plastic tube of the same size is the alternative. | 1 to 2 |  |  |
| [ ] | Silhouette film | Black adhesive vinyl sheet, or transparent printable adhesive film, to cut or print the fairy silhouette that shows when the candle is lit from behind. Use a fairy portrait of yours for the shape. | 1 pack |  |  |
| [ ] | Fixings | Hot glue, double sided foam tape, a few small screw terminal blocks, jumper wires. Support every wire so nothing hangs from a joint. | 1 set |  |  |

## Do not buy yet

A battery and a charging circuit. The first candle runs from a cord.

The base that charges the candles and lets them talk to each other. Decide first how many candles it holds.

The printed fairy cards and the packaging. Wait until the portraits and lore are final.

More than one candle shell. Test one first, and look at how the silhouette and the light look together.

# The order to buy in

The light test parts, Part 1 "Buy now". Do the light test first. Nothing else depends on anything not yet decided.

The audio board and the speaker, together, since both the light and the candle use them.

The candle body, the flame LED, the button and the film, after the audio works and a hello can be heard.

Everything under "Do not buy yet", one at a time, as each decision is made.

# Things I am not sure of

Whether the audio board you find has a free pin for the LED. If it shows none, tell me before buying.

Whether the Astra-9 speakers fit inside a 3 inch candle. Measure one.

The speakers and the LED power wires can make a buzz, as the Astra-9 eye lights did. The cause there is not fully known.

The ESP32-S3 has Wi-Fi and low energy Bluetooth but not the regular Bluetooth that phone speakers use. Music comes through Wi-Fi from Apple Music or Spotify.